WEATHER MONITORING SYSTEM USING NODE MCU

A PROJECT REPORT

Submitted by

MAHARAJAN M (19BCS089)

SIVA PRAKASH (19BCS073)

BHARATH SANKAR (19BCS076)

PRAVEEN (19BCS094)

COMPUTER SCIENCE AND ENGINEERING

KUMARAGURU COLLEGE OF TECHNOLOGY

COIMBATORE-641 049

(An Autonomous Institution Affiliated to Anna University, Chennai)

DECEMBER 2021
KUMARAGURU COLLEGE OF TECHNOLOGY

COIMBATORE 641 049

(An Autonomous Institution Affiliated to Anna University, Chennai)

BONAFIDE CERTIFICATE

Certified that this project report “WEATHER MONITORING SYSTEM USING NODE MCU” is the Bonafide work of

(“MAHARAJAN M (19BCS089),SIVA PRAKASH (19BCS073),BHARATHSANKAR (19BCS076),PRAVEEN (19BCS094)”)who carried out the project work under my supervision.

| SIGNATURE | SIGNATURE |
| --- | --- |
| Dr. Devaki. P, Ph.D., | Senthil Kumar V, Assistant Professor |
| HEAD OF THE DEPARTMENT | SUPERVISOR |
| Department of Computer Science and Engineering, | Department of Computer Science and Engineering, |
| Kumaraguru College of Technology | Kumaraguru College of Technology |
| Coimbatore – 641 049 | Coimbatore – 641 049 |

The candidates with University register number 19BCS089, 19BCS073, 19BCS076,19BCS094 were examined in the Project Viva-Voce examination held on …………………

Internal Examiner								External Examiner

DECLARATION

We affirm that the project work titled “WEATHER MONITORING SYSTEM USING NODE MCU AND DHT 11 SENSOR” is being completed for the Course  ENGINEERING CLINICS – V . It has not formed the part of any other project work submitted for any other course.

MAHARAJAN M (19BCS089)                     SIVA PRAKASH (19BCS072)

BHARATH SANKAR (19BCS076)             PRAVEEN (19BCS094)

I certify that the declaration made above by the candidates is true.

Senthil Kumar V,

Assistant Professor,

Department of Computer Science and Engineering,

Kumaraguru College of Technology,

Coimbatore – 641 049.

ACKNOWLEDGEMENT

We express our profound gratitude to the management of Kumaraguru College of Technology for providing as with the required infrastructure that enabled us to successfully complete the project.

We extend our gratefulness to our Principal, Dr.J. Srinivasan, for providing us the necessary facilities to pursue the project.

We would like to acknowledge Dr. P. Devaki, Professor and Head, Department of Computer Science and Engineering, for her support and encouragement throughout this project.

We thank our Engineering clinics faculty Prof.V.Senthil Kumar, Assistant Professor, Department of Computer Science and Engineering, for their constant and continuous effort, guidance and valuable time.

- MAHARAJAN M (19BCS089)

SIVA PRAKASH (19BCS072)

BHARATH SANKAR (19BCS076)

PRAVEEN(19BCS094)

### TABLE OF CONTENTS

| CHAPTER NO. | TITLE |  | PAGE NO. |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ABSTRACT |  | 2 |  |  |
| 1 | INTRODUCTION |  | 3 |  |  |
| 2 | LITERATURE REVIEW |  | 5 |  |  |
| 3 | PROBLEM DEFINITION |  | 8 |  |  |
| 4 | PROPOSED SYSTEM |  | 9 |  |  |
|  |  | 4.1 IMPLEMENTATION-INSTALLING REQUIREMENTS 4.2 IMPLEMENTING RESULTS |  | 11 12 |
| 5 | SYSTEM REQUIREMENTS |  | 15 |  |  |
|  |  | 5.1 HARDWARE REQUIREMENTS 5.2 SOFTWARE REQUIREMENTS | 15 15 |  |  |
| 6 | CONCLUSION |  | 16 |  |  |
| 7 | REFERENCES |  | 17 |  |  |

8                   SAMPLE CODE                                                                           18

GITHUB REPOSITORY                                                              20

Abstract

The system proposed is an advanced solution for monitoring the weather conditions at a particular place and make the information visible anywhere in the world. The technology behind this is Internet of Things (IoT), which is an advanced and efficient solution for connecting the things to the internet and to connect the entire world of things in a network. Node MCU, sensors and automotive electronic equipment. The system deals with monitoring and controlling the environmental conditions like temperature, relative humidity and CO level with sensors and sends the information to the web page and then plot the sensor data as graphical statistics. The data updated from the implemented system can be accessible in the internet from anywhere in the world.

CHAPTER - 1. INTRODUCTION

Present day innovations in technology mainly focus on controlling and monitoring of different devices over wirelessly over the internet such that  the internet acts as a medium  for communication between  all the  devices. Most of this technology is focused on efficient monitoring and controlling of different. An efficient environmental monitoring system is required to monitor and assess the weather conditions in case of exceeding the prescribed level of parameters (e.g., noise, CO and  radiation levels) and for gathering data for research purposes (amount of  rainfall, windspeed etc.).

A system is considered as a smart system when the device equipped with sensors, microcontrollers and various software applications becomes a self-protecting and self-monitoring system. The recent changes in climate have increased the importance of environment monitoring. The current advance in the field of technology and economy are hardly a significant impact over the environment, have led to  serious concern regarding pollution and climate change. The intergovernmental panel on climate change (IPCC) in the report confirms that human activities are having an unequivocal and continuously increasing influence on  the climate  system, with recent changes that are unprecedented over decades of millennia.

In this contest, environmental monitoring  represents a fundamentals instrument for  gathering relevant information about the ecosystem, leading to new knowledge and understanding and for ultimately adaptation and mitigation actions that address the degradation of the biosphere. Event  Detection based and  Spatial Process Estimation  are  the two categories  to which applications are  classified. Initially  the  sensor  devices  are  deployed  in  environment  to  detect  the  parameters  (e.g.,  Temperature,  Humidity, Pressure, LDR, noise, CO and radiation levels etc.) while the data acquisition, computation and controlling action (e.g., the variations in the noise and CO levels with respect to the quantified levels). Sensor devices are positioned at different locations to  collect the  data to  forecast the  behavior of a particular area  of interest. The  main aim  of this  paper is  to design and implement a resourceful monitoring system through which the required parameters are monitored remotely using internet and the data gathered from the devices are stored in the cloud and to project the predictable trend on the web browser. A solution for monitoring temperature and CO levels i.e., any parameter value crossing its threshold value ranges, for example  CO levels in air  in  a particular area exceedingly the  normal levels etc., in  the  atmosphere  using wireless embedded computing system is proposed in this paper. The solution also provides an intelligent remote monitoring for a particular area of interest. In this paper we also current results of collected or sensed data with respect to the normal or  specified  ranges  of  particular  parameters. The  embedded  system  is  an  integration  of  sensor  devices,  wireless communication which enables the user to remotely access the various parameters and store the data in cloud.

CHAPTER - 2. LITERATURE REVIEW

[1] Weather monitoring and forecasting system using IoT

Balakrishnan Sivakumar and Chikkamadaiah Nanjundaswamy(GJETA)

This Paper says that in today’s world many pollution monitoring  systems are designed  by different environmental  parameters. Existing system  model  is presented  IOT  based  Weather  monitoring and  reporting system  where  you  can  collect, process, analyses, and present your measured data on web server. Wireless sensor network management model consists of and device, router, gateway node and management monitoring center. End  device is responsible for collecting wireless sensor network data, and sending them to parent node, the data are sent to gateway node from parent node directly or by  router. After receiving the data  from  wireless  sensor network, gateway node extracts  data  after analysing and packaging them into Ethernet format data, sends them to the server. Less formally, any device that runs server software could  be considered a server as well. Servers are used to manage network  resources. The services or information provided through the Internet that are connected  through LAN and made available for users via smart phones, web browser or other web browser devices to make the system more intelligent, adaptable and efficient.

[2] Smart weather monitoring and real time alert weather and climate changes like system using IoT

Yashaswi Rahut , Rimsha Afreen, Divya Kamini, SRM Institute of Science and temperature, humidity, wind Technology. IRJET - vol. 05, issue 10, Oct 2018.

This paper says that IOT Based Weather Monitoring System is the network of physical devices that allow the devices to communicate with each other. IOT allows remote sensing and control over the devices. It is an advanced automation and analysis system which uses its sensors to deliver advanced and automated products and services. These systems allow greater transparency, control, and good performance. IOT has several automation applications like smart home, smart parking, smart roads etc. Weather is the day-to-day state of the atmosphere that is hard to predict which affects the activities of mankind and has great significance in many different domains. However, the current weather station in the market is expensive and bulky which cause inconvenience. The aim of this project is to design a weather station with real time notifications for climatology monitoring, interface it to a cloud platform and analyse weather parameters. Existing system: The existing weather monitoring systems generally use weather stations that use multiple instruments such as thermometers, barometers, wind vanes, rain gauge etc. to measure weather and climate changes. Most of these instruments use simple analog technology which is later physically recorded and stored in a database.

[3] Development of IoT Based Weather Reporting System

A F Pauzi and M Z Hasan, International Conference on Technology, Engineering and Sciences (ICTES) 2020

This paper says that End gadget is liable for gathering remote sensor network information, and sending parent hub straightforwardly or by switch. Subsequent to getting the information from remote sensor organization, entryway hub extricates information in the wake of dissecting and bundling them into Ethernet design information, sends them to the worker. Less officially, any gadget that runs worker programming could be viewed as a worker also. In IOT empowered climate checking framework project, Arduino Uno estimates four climate boundaries utilizing four separate sensors. Arduino has inbuilt Analog to computerized converter. Arduino figures and shows these climate boundaries on LCD show. At that point the client needs to visit a specific site to see the climate information.

3. PROBLEM DEFINITION

The satellite weather reporting system provides the current condition that does not give the exact location condition. The drawbacks are in conventional approach where the devices are costly and have no visualization of information. There is no such automatic tool to offer the alert signal in case of any abnormalities, so it is difficult to regulate this abnormality.

CHAPTER - 4. PROPOSED SYSTEM

This IoT based project has four sections. Firstly DHT11 sensor senses the Humidity & Temperature Data.

Secondly Node MCU reads the DHT11 sensor module’s output by using single wire protocol into a suitable number in percentage (humidity), Celsius scale (temperature).

Thirdly, these values are sent to ThingSpeak server by using inbuilt Wi-Fi of Node MCU. And finally ThingSpeak analyses the data and shows it in a Graph form.

In Arduino (Version1.8.16)  Software we have installed a Port (COM5) for displaying the weather Conditions

NODE MCU AND DHT 11 SENSOR ARE INTEGRATED WITH THINGS SPEAK SERVER.

IMPLEMENTATION

CHAPTER - 4. 1. INSTALLING THE REQUIREMENTS

- Arduino IDE 1.8.16 - The open-source Arduino Software (IDE) makes it easy to write code and upload it to the board. This software can be used with any Arduino board.

- Embedded C - It is a set of language extensions for the C programming language by the C Standards Committee to address commonality issues that exist between C extensions for different embedded systems.

- DHT.h – It’s a library has all the functions needed to get the humidity and temperature readings from the sensor.

- ESP8266.h - This library provides ESP8266 specific Wi-Fi routines that we are calling to connect to the network. The actual connection to Wi-Fi is initialized by calling: begin("network-name", "pass-to-network").

CHAPTER - 4.2 IMPLEMENTATION RESULTS

CHAPTER - 5. SYSTEM REQUIREMENTS

CHAPTER - 5.1.Hardware Requirement:

NODE MCU

Humidity & Sensor(DHT 11)

BREADBOARD

JUMPER WIRES

Internet connection

CHAPTER-5.2.Software Requirement:

THINGS SPEAK(API GENERATOR WEB.WE CAN INTEGRATE IT WITH NODE MCU)

PROGRAMMING LANGUAGE:EMBEDDED C

CHAPTER - 6. CONCLUSION

In this paper, we proposed an approach that to  implement this  need  to deploy  the  sensor devices in  the  environment  for collecting the  data and analysis.  By deploying sensor devices in the environment, it will record real time data. It can cooperate with other objects through the network. Then the collected data and analysis results will be available to the end user through the Wi-Fi. The smart way to  monitor environment and an efficient, low-cost entrenched system is presented with different models in this paper.

Weather station is a  form of  application of  science and technology  to know  and predict  the weather  condition at  a particular body. The weather station is designed to collect quantitative data about the weather condition of a region in order to be able to known and predict weather conditions in a region . Human activities depend on whether factors.

Additions that can be made to improve the system

1)Powering the device using solar panels.

2) Using silica gel to prevent condensation on the exterior cover as condensation might affect the sensors readings.

CHAPTER - 7.REFERENCES

[1]  Weather monitoring and forecasting system using IoT

Balakrishnan Sivakumar and Chikkamadaiah Nanjundaswamy(GJETA)

[2] Development of IoT Based Weather Reporting System

A F Pauzi and M Z Hasan, International Conference on Technology, Engineering and Sciences (ICTES) 2020

[3] Internet of Things (IOT) based Weather Monitoring System

Girija C, Andreanna Grace Shires, Harshalatha H , Pushpalatha H P , International Journal of Engineering Research & Technology (IJERT) 2018

[4] Smart weather monitoring and real time alert weather and climate changes like system using IoT

Yashaswi Rahut , Rimsha Afreen, Divya Kamini, SRM Institute of Science and temperature, humidity, wind Technology. IRJET - vol. 05, issue 10, Oct 2018.

[5] Environment Monitoring System using Node Mcu / Raspberry Pi

Gaurav Jadhav, Kunal Jadhav, Kavita Nadlamani , R.H. Sapat College of Engineering, Management Studies and Research. IRJET - vol. 3, issue 4, Apr 2016.

CHAPTER – 8. SAMPLE CODE:-

#include <DHT.h>

#include <ESP8266WiFi.h>

String apiKey = "VN8MQXZM5B0YU5ZE";     //  Enter your Write API key here

const char *ssid =  "vivo 1818";     // Enter your WiFi Name

const char *pass =  "rajaraja"; // Enter your WiFi Password

const char* server = "api.thingspeak.com";

#define DHTPIN 4          // GPIO Pin where the dht11 is connected

DHT dht(DHTPIN, DHT11);

WiFiClient client;

void setup()

{

Serial.begin(115200);

delay(10);

dht.begin();

Serial.println("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED)

{

delay(550);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected");

}

void loop()

{

float h = dht.readHumidity();

float t = dht.readTemperature();

if (isnan(h) || isnan(t))

{

Serial.println("Failed to read from DHT sensor!");

return;

}

if (client.connect(server,80))

{

String postStr = apiKey;

postStr +="&field1=";

postStr += String(t);

postStr +="&field2=";

postStr += String(h);

postStr += "\r\n\r\n";

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("Connection: close\n");

client.print("X-THINGSPEAKAPIKEY: "+apiKey+"\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

Serial.print("Temperature: ");

Serial.print(t);

Serial.print(" degrees Celcius, Humidity: ");

Serial.print(h);

Serial.println("%. Send to Thingspeak.");

}

client.stop();

Serial.println("Waiting...");

delay(10000);

}

OUR GITHUB REPOSITORY:

https://github.com/maharajan456/WEATHER-MONITORING-SYSTEM